http://www.ddrservice.net/f0z/Integrated-circuits/F/fi1216.pdf

; FI1216 control, ADC and LED bar driver

| ; | V0.0 | 141130 | Initial draft |
| --- | --- | --- | --- |
| ; | V0.1 | 141201 | Done ADC, LED bar |
| ; | V0.2 | 141202 | Subroutines, TMR0 |
| ; | V0.3 | 141203 | Tetra 4s, MAX HOLD function |
| ; | V0.4 | 141204 | Fixed AD, lose controls, enter buttons |
| ; | V0.5 | 141206 | I2C |
| ; | V0.6 | 141207 | Fixed I2C |
| ; | V0.7 | 141228 | BRT Button |
| ; | V0.8 | 141231 | THR Button |
| ; | V0.9 | 150118 | Cal levels FI1216, f(LO) |
| ; | V0.10 | 150119 | THR setting |
| ; | V0.11 | 150119 | EEPROM R/W |
| ; | V1.0 | 150119 LIST | Release P=16F818, F=INHX8M |

#include <p16f818.inc>

CONFIG _WDT_OFF & _PWRTE_OFF & _INTRC_IO & _MCLR_ON & _BODEN_OFF & _LVP_OFF & _CPD_OFF &

_WRT_ENABLE_OFF & _DEBUG_OFF & _CCP1_RB2 & _CP_OFF

; Equates

RESET_V	EQU	0x00	; Address of RESET Vector

OSC_FREQ	EQU	D'8000000'	; Internal Oscillator Frequency is 8 MHz

; Registers

| FLAGS | EQU | 0x20 |  | ; | Various flags |
| --- | --- | --- | --- | --- | --- |
| THRLVL | EQU | 0x21 |  | ; | Threshold level set via UI |
| MAXH | EQU | 0x22 |  | ; | LEVEL MAX HOLD value to be shown on LED bar |
| TEMPBAR | EQU | 0x23 |  | ; | Pattern on LED bar if it's blanked (BRT) |
| TEMPTHR | EQU | 0x24 |  | ; | Used in threshold set routine |
| BRITE | EQU | 0x25 |  | ; | LED Brightness |
| T4S | EQU | 0x26 |  | ; | Tetra 4s timing |
| I2CDELAY |  | EQU | 0x27 | ; | Used for I2C timing |
| I2CBUF | EQU | 0x28 |  | ; | Used for I2C data |
| I2CCNT | EQU | 0x29 |  | ; | Used for I2C bitbanger |
| DB2 | EQU | 0x30 |  | ; | Stealth byte for prescaler (0x50-0x5F) |
| TEMPW | EQU | 0x40 |  | ; | Context saving for interrupts |

TEMPSTATUS	EQU	0x41	; in all banks

; Defines

#define LEDS	PORTB	; 8 LED bar RB0-3 GRN, RB4,5 ORG RB6,7 RED

#define RSSI	PORTA,0	; A/D input from AD8307 #define ALARM	PORTA,1	; Threshold Alarm Output #define THR	PORTA,3	; Alarm threshhold button input #define BRT	PORTA,4	; LED brightness button input #define SCL	PORTA,6

#define SDA	PORTA,7	; I2C bus outputs #define BLANKBAR FLAGS,0	; 1: blanked (BRT) #define BRTBTN FLAGS,1	; 1: BRT Button Pressed #define NITE	FLAGS,3	; 1: lowest brightness #define DIM	FLAGS,4	; 1: low brightness #define DAY	FLAGS,5	; 1: mid brightness #define FULL	FLAGS,6	; 1: max brightness

ORG	0

GOTO	START

ORG	4

; Timer0 Interrupt Handler

BCF	INTCON,TMR0IF	; Clear TMR0 interrupt

BCF	INTCON,GIE	; Disable global interrupts

MOVWF	TEMPW	; Copy W to a Temporary Register (all banks) SWAPF	STATUS,W	; Swap STATUS nibbles and place into W register MOVWF	TEMPSTATUS	; Save STATUS to a Temporary register (DS31008a) CALL	BARBRITE	; BRT Button test

CALL	THRSET	; Threshold Setting Routine

MOVLW	B'00000001'

XORWF	FLAGS,1	; XOR BLANKBAR

BTFSC	BLANKBAR	; Test if clear

GOTO	BLANKLEDS	; Set, blank leds

MOVFW	BRITE	; Fetch LED brightness

SUBLW	0xFF	; Subtract BRITE from 0xFF

MOVWF	TMR0	; And load Timer0

MOVFW	TEMPBAR	; Fetch LED pattern

MOVWF	LEDS	; Write to LEDs

GOTO	ENDINT BLANKLEDS

| MOVFW | BRITE | ; | Fetch LED brightness |
| --- | --- | --- | --- |
| MOVWF | TMR0 | ; | And load Timer0 |
| CLRF | LEDS | ; | Switch off LEDs |

| ENDINT | CALL | TETRA4S | ; | Check if 4s have elapsed, if so write pattern to TEMPBAR. |
| --- | --- | --- | --- | --- |
|  | SWAPF | TEMPSTATUS,W | ; | Swap original STATUS register value |
|  | MOVWF | STATUS | ; | Restore STATUS register from W |
|  | SWAPF | TEMPW,F | ; | Swap W_Temp nibbles and return value to W_Temp |
|  | SWAPF | TEMPW,W | ; | Swap W_Temp to W to restore original W (DS31008a) |
|  | BSF | INTCON,GIE | ; | Re-enable global interrupts |
|  | RETFIE |  |  |  |
| START |  |  |  |  |
| ; Init | stuff |  |  |  |
|  | CLRF | STATUS | ; | Do initialization, Select bank 0 |
|  | CLRF | INTCON |  |  |
|  | BSF | INTCON,GIE | ; | Enable GIE |
|  | BSF | INTCON,TMR0IE | ; | Enable Timer0 Interrupt |
|  | CLRF | PCLATH | ; | Keep in lower 2KByte |
|  | CLRF | CCP1CON |  |  |
|  | MOVLW | B'00111111' |  |  |
|  | BANKSEL | TRISA |  |  |
|  | MOVWF | TRISA | ; | RA7,6 Outputs, 5-0 Inputs |
|  | CLRF | TRISB | ; | RB7-0 Outputs |
|  | MOVLW | B'00000110' | ; | Timer0, prescaler 1:128 |
|  | MOVWF | OPTION_REG |  |  |
|  | MOVLW | B'01110000' | ; | 8 MHz clock |
|  | MOVWF | OSCCON |  |  |
|  | MOVLW | B'00001110' | ; | AN0 Analog input, AN1-4 Digital IO, ADRESH only (ADRESL discarded) |
|  | MOVWF | ADCON1 |  |  |
|  | MOVLW | B'01000001' | ; | AD conv ON, AN0 Selected, Fosc/8 (T(AD)=1us) |
|  | BANKSEL | ADCON0 |  |  |
|  | MOVWF | ADCON0 |  |  |
|  | CLRF | PORTB | ; | Make all PORT B outputs low |
|  | CLRF | TMR0 | ; | Reset Timer 0 |
|  | MOVLW | 0x04 | ; | Startup Brightness ("dim") |
|  | MOVWF | BRITE |  |  |
|  | BSF | DIM |  |  |
| ; Restore Threshold from EEPROM |  |  |  |  |
|  | BANKSEL | EEADR | ; | Select Bank of EEADR |
|  | MOVLW | 0x00 |  |  |
|  | MOVWF | EEADR | ; | Data Memory Address to read (0x00) |
|  | BANKSEL | EECON1 | ; | Select Bank of EECON1 |
|  | BCF | EECON1, EEPGD | ; | Point to Data memory |
|  | BSF | EECON1, RD | ; | EE Read |
|  | BANKSEL | EEDATA | ; | Select Bank of EEDATA |
|  | MOVF | EEDATA, W | ; | W = EEDATA |
|  | BANKSEL | THRLVL |  |  |
|  | MOVWF | THRLVL | ; | Restored Threshold |
|  | BSF | SDA | ; | I2C idle |
|  | BSF | SCL |  |  |
|  | MOVLW | 0x60 | ; | Set Prescaler second byte (stealth mode) |
|  | MOVWF | DB2 |  |  |
|  | CALL | DEBOUNCE | ; | Clock Stabilization delay 16 ms |
|  | CALL | TUNE | ; | Set tuner frequency |
| MAIN |  |  |  |  |
|  | BSF | ADCON0,GO | ; | Start A/D (RSSI) |
| CONV0 |  |  |  |  |
|  | NOP |  |  |  |
|  | BTFSC | ADCON0,GO | ; | Test if done |
|  | GOTO | CONV0 |  |  |
|  | MOVFW | ADRESH | ; | Move 8 bit A/D result to W |
|  | SUBWF | MAXH,0 | ; | Subtract W from MAXH |
|  | BTFSC | STATUS,C | ; | result larger than MAXH? |
|  | GOTO | MAIN | ; | No |
|  | MOVFW | ADRESH |  |  |
|  | MOVWF | MAXH | ; | New MAXH |
|  | GOTO | MAIN |  |  |

TETRA4S

; Subroutine

; Tests if 4 seconds have elapsed

; Converts MAXH to LED bar

DECFSZ T4S,1

# RETURN

MOVLW	0xF2	; 0xF2 for 4s

MOVWF	T4S

CALL	LEDBAR

MOVWF	TEMPBAR

CLRF	MAXH	; Destroy Max Hold value

CALL	CHECKTHR	; Check threshold, beep if reached

# RETURN

LEDBAR

; MAXH contains RSSI max value

; LED bar pattern is in W

MOVFW	MAXH

ADDLW	0x9A	; LEVEL > -40 dBm

BTFSS	STATUS,C GOTO	LEDS7F

LEDS7F

LEDS3F

LEDS1F

LEDS0F

LEDS07

LEDS03

LEDS01

LEDS00

MOVLW	0xFF	; Fullscale

# RETURN

MOVFW	MAXH

ADDLW	0xA5	; LEVEL > -48 dBm

BTFSS	STATUS,C GOTO	LEDS3F

MOVLW	0x7F

# RETURN

MOVFW	MAXH

ADDLW	0xAF	; LEVEL > -56 dBm

BTFSS	STATUS,C GOTO	LEDS1F

MOVLW	0x3F

# RETURN

MOVFW	MAXH

ADDLW	0xBA	; LEVEL > -64 dBm

BTFSS	STATUS,C GOTO	LEDS0F

MOVLW	0x1F

# RETURN

MOVFW	MAXH

ADDLW	0xC5	; LEVEL > -72 dBm

BTFSS	STATUS,C GOTO	LEDS07

MOVLW	0x0F

# RETURN

MOVFW	MAXH

ADDLW	0xD0	; LEVEL > -80 dBm

BTFSS	STATUS,C GOTO	LEDS03

MOVLW	0x07

# RETURN

MOVFW	MAXH

ADDLW	0xDA	; LEVEL > -88 dBm

BTFSS	STATUS,C GOTO	LEDS01

MOVLW	0x03

# RETURN

MOVFW	MAXH

ADDLW	0xE3	; LEVEL > -96 dBm

BTFSS	STATUS,C GOTO	LEDS00

MOVLW	0x01

# RETURN

MOVLW	0x00	; LEVEL at noise floor -108 dBm

# RETURN

| ; Set frequency TUNE | of tuner |  |  |
| --- | --- | --- | --- |
| BCF | INTCON,GIE | ; | Disable global interrupts |
| CALL | I2CSTART |  |  |
| MOVLW | 0xC0 | ; | Address |
| CALL | I2CBYTE |  |  |
| MOVLW | 0x34 | ; | Prescaler first byte |
| CALL | I2CBYTE |  |  |
| MOVFW | DB2 | ; | Get Prescaler second byte |
| CALL | I2CBYTE |  |  |
| MOVLW | 0xCA | ; | Control Byte |

|  | CALL | I2CBYTE |  |  |
| --- | --- | --- | --- | --- |
|  | MOVLW | 0x90 | ; | Port Byte (Mid Band) |
|  | CALL | I2CBYTE |  |  |
|  | CALL | I2CSTOP |  |  |
|  | BCF | INTCON,TMR0IF | ; | Clear TMR0 interrupt just in case |
|  | BSF | INTCON,GIE | ; | Enable global interrupts |
|  | RETURN |  |  |  |
| I2CSTART |  |  |  |  |
|  | BCF | SDA |  |  |
|  | CALL | DELAY10US |  |  |
|  | BCF | SCL |  |  |
|  | CALL | DELAY10US |  |  |
|  | RETURN |  |  |  |
| I2CBYTE |  |  |  |  |
|  | MOVWF | I2CBUF |  |  |
|  | MOVLW | 0x08 |  |  |
|  | MOVWF | I2CCNT |  |  |
| NEXTBIT |  |  |  |  |
|  | BCF | STATUS,C | ; | Clear carry |
|  | BCF | SDA | ; | Glitch if next bit is 1 again UGLY! |
|  | RLF | I2CBUF,1 |  |  |
|  | BTFSC | STATUS,C |  |  |
|  | BSF | SDA |  |  |
|  | CALL | DELAY10US |  |  |
|  | BSF | SCL | ; | Send clock |
|  | CALL | DELAY10US |  |  |
|  | BCF | SCL |  |  |
|  | CALL | DELAY10US | ; | Done clock |
|  | DECFSZ | I2CCNT,1 |  |  |
|  | GOTO | NEXTBIT |  |  |
|  | BCF | SDA | ; | Fake ACK |
|  | CALL | DELAY10US |  |  |
|  | BSF | SCL |  |  |
|  | CALL | DELAY10US |  |  |
|  | BCF | SCL |  |  |
|  | CALL | DELAY10US |  |  |
|  | RETURN |  |  |  |
| I2CSTOP |  |  |  |  |
|  | BCF | SDA |  |  |
|  | CALL | DELAY10US |  |  |
|  | BSF | SCL |  |  |
|  | CALL | DELAY10US |  |  |
|  | BSF | SDA |  |  |
|  | RETURN |  |  |  |
| DELAY10US |  |  |  |  |
| MOVLW |  | 0x14 | ; | 33 us delay |
| MOVWF |  | I2CDELAY |  |  |

LOOP0

DECFSZ I2CDELAY,1 GOTO	LOOP0 RETURN

DEBOUNCE

; Subroutine, used in THR setting and INIT

MOVLW	0xFF	; 96 ms outer delay

MOVWF	I2CCNT

LOOP1 LOOP2

MOVLW	0xFF	; 0.39 ms inner delay

MOVWF	I2CDELAY

DECFSZ I2CDELAY,1 GOTO	LOOP2 DECFSZ I2CCNT,1 GOTO	LOOP1 RETURN

BARBRITE

; Subroutine (called from within INT handler)

; Modifies BAR brightness after BRT is pressed

; Debounce (every 16 ms)

| BTFSC GOTO | BRT NOBRT | ; Test BRT Button |
| --- | --- | --- |
| BTFSC RETURN | BRTBTN | ; BRT Button still pressed? |
| BSF | BRTBTN |  |
| BTFSC | NITE |  |
| GOTO | DIMBRT |  |

|  | BTFSC | DIM |
| --- | --- | --- |
|  | GOTO | DAYBRT |
|  | BTFSC | DAY |
|  | GOTO | FULLBRT |
|  | BCF | FULL |
|  | BSF | NITE |
|  | MOVLW | 0x01 |
|  | MOVWF RETURN | BRITE |
| DIMBRT | BCF | NITE |
|  | BSF | DIM |
|  | MOVLW | 0x04 |
|  | MOVWF RETURN | BRITE |
| DAYBRT | BCF | DIM |
|  | BSF | DAY |
|  | MOVLW | 0x20 |
|  | MOVWF RETURN | BRITE |
| FULLBRT | BCF | DAY |
|  | BSF | FULL |
|  | MOVLW | 0xF0 |
|  | MOVWF RETURN | BRITE |
| NOBRT | BCF | BRTBTN |
|  | RETURN |  |

THRSET

; Subroutine (called from within INT handler)

; User interface to change threshold for alarm

; All LEDs are full on because the PWM brightness control uses the interrupt

|  | BTFSC | THR | ; | Test THR Button |
| --- | --- | --- | --- | --- |
|  | GOTO | NOTHR |  |  |
| THR1 | CLRF | LEDS | ; | switch off LEDs |
|  | CALL | DEBOUNCE | ; | 0.1 s delay |
|  | BTFSS | THR | ; | THR Button still pressed? |
|  | GOTO | THR1 |  |  |
| THR2 | MOVFW | THRLVL |  |  |
|  | MOVWF | LEDS | ; | Show threshold |
|  | MOVWF | TEMPTHR |  |  |
| THR3 | BTFSS | THR | ; | THR button? |
|  | GOTO | DECTHR |  |  |
|  | BTFSS | BRT | ; | BRT button? |
|  | GOTO | INCTHR |  |  |
|  | GOTO | THR3 |  |  |

; INC threshold INCTHR

| CLRF | LEDS | ; | switch off LEDs |
| --- | --- | --- | --- |
| CALL | DEBOUNCE |  |  |
| CALL | DEBOUNCE | ; | 0.2 s delay |
| MOVFW | THRLVL |  |  |
| SUBLW | 0xFF | ; | FF? |
| BTFSS | STATUS,Z |  |  |
| GOTO | INC7F |  |  |
| GOTO | DONETHR | ; | yes, exit INC |

INC7F

INC3F

INC1F

MOVFW	THRLVL

SUBLW	0x7F BTFSS	STATUS,Z GOTO	INC3F

MOVLW	0xFF

MOVWF	THRLVL

GOTO	DONETHR

MOVFW	THRLVL

SUBLW	0x3F BTFSS	STATUS,Z GOTO	INC1F

MOVLW	0x7F

MOVWF	THRLVL

GOTO	DONETHR

MOVFW	THRLVL

SUBLW	0x1F BTFSS	STATUS,Z GOTO	INC0F

|  | MOVLW | 0x3F |
| --- | --- | --- |
|  | MOVWF | THRLVL |
|  | GOTO | DONETHR |
| INC0F | MOVFW | THRLVL |
|  | SUBLW | 0x0F |
|  | BTFSS | STATUS,Z |
|  | GOTO | INC07 |
|  | MOVLW | 0x1F |
|  | MOVWF | THRLVL |
|  | GOTO | DONETHR |
| INC07 | MOVFW | THRLVL |
|  | SUBLW | 0x07 |
|  | BTFSS | STATUS,Z |
|  | GOTO | INC03 |
|  | MOVLW | 0x0F |
|  | MOVWF | THRLVL |
|  | GOTO | DONETHR |
| INC03 | MOVLW | 0x07 |
|  | MOVWF | THRLVL |
|  | GOTO | DONETHR |

| CLRF | LEDS | ; switch off LEDs |
| --- | --- | --- |
| CALL | DEBOUNCE |  |
| CALL | DEBOUNCE | ; 0.2 s delay |
| MOVFW | THRLVL |  |
| SUBLW | 0xFF | ; FF? |
| BTFSS | STATUS,Z |  |
| GOTO | DEC7F |  |
| MOVLW | 0x7F |  |
| MOVWF | THRLVL |  |
| GOTO | DONETHR |  |
| MOVFW | THRLVL |  |
| SUBLW | 0x7F | ; 7F? |
| BTFSS | STATUS,Z |  |
| GOTO | DEC3F |  |
| MOVLW | 0x3F |  |
| MOVWF | THRLVL |  |
| GOTO | DONETHR |  |
| MOVFW | THRLVL |  |
| SUBLW | 0x3F |  |
| BTFSS | STATUS,Z |  |
| GOTO | DEC1F |  |
| MOVLW | 0x1F |  |
| MOVWF | THRLVL |  |
| GOTO | DONETHR |  |
| MOVFW | THRLVL |  |
| SUBLW | 0x1F |  |
| BTFSS | STATUS,Z |  |
| GOTO | DEC0F |  |
| MOVLW | 0x0F |  |
| MOVWF | THRLVL |  |
| GOTO | DONETHR |  |
| MOVFW | THRLVL |  |
| SUBLW | 0x0F |  |
| BTFSS | STATUS,Z |  |
| GOTO | DEC07 |  |
| MOVLW | 0x07 |  |
| MOVWF | THRLVL |  |
| GOTO | DONETHR |  |
| MOVFW | THRLVL |  |
| SUBLW | 0x07 |  |
| BTFSS | STATUS,Z |  |
| GOTO | DONETHR |  |
| MOVLW | 0x03 |  |
| MOVWF | THRLVL |  |

; DEC threshold DECTHR

DEC7F

DEC3F

DEC1F

DEC0F

DEC07

; There's no lower threshold, that'd be silly

; Done manipulating threshold DONETHR

| BTFSC | THR | ; THR button? |
| --- | --- | --- |
| GOTO | THR2 | ; No |
| MOVFW | THRLVL |  |
| CALL | DEBOUNCE | ; 0.1 s delay |
| BTFSC | THR | ; Still THR button? |

| GOTO | THR2 | ; | Yes, exit and restore THR |
| --- | --- | --- | --- |
| CLRF | LEDS | ; | switch off LEDs |
| MOVFW | TEMPTHR | ; | Restore THR |
| MOVWF | THRLVL |  |  |
| THR4 CALL | DEBOUNCE | ; | to enable releasing THR button |
| BTFSS | THR |  |  |
| GOTO | THR4 |  |  |
| CALL | DEBOUNCE |  |  |

; Store THRLVL into EEPROM (copied from datasheet) BANKSEL EECON1	; Select Bank of EECON1 BTFSC	EECON1, WR	; Wait for write

GOTO	$-1	; to complete

BANKSEL EEADR	; Select Bank of EEADR

MOVLW	0x00

MOVWF	EEADR	; Data Memory Address to write

BANKSEL THRLVL MOVFW THRLVL BANKSEL EEDATA

MOVWF	EEDATA	; Data Memory Value to write

BANKSEL EECON1	; Select Bank of EECON1 BCF	EECON1, EEPGD	; Point to DATA memory BSF	EECON1, WREN	; Enable writes

MOVLW	0x55

MOVWF	EECON2	; Write 55h

MOVLW	0xAA

MOVWF	EECON2	; Write AAh

BSF	EECON1, WR	; Set WR bit to begin write

BCF	EECON1, WREN	; Disable writes

BANKSEL PORTB	; Select BANK0

NOTHR

# RETURN

CHECKTHR

; Subroutine called from TETRA4S

; Checks if threshold is reached

MOVFW	TEMPBAR SUBWF	THRLVL,0 BTFSC	STATUS,Z GOTO	PIP RETURN

| MOVLW BANKSEL | B'00111101' TRISA |  |  |
| --- | --- | --- | --- |
| MOVWF | TRISA | ; | RA7,6,1 Outputs, 5-2,0 Inputs (ALARM output) |
| MOVLW BANKSEL MOVWF | 0xAA I2CCNT I2CCNT | ; | 64 ms outer delay |
| MOVLW | 0xFF | ; | 0.39 ms inner delay |
| MOVWF | I2CDELAY |  |  |
| DECFSZ GOTO | I2CDELAY,1 LOOP4 |  |  |
| MOVLW | B'00000010' |  |  |
| XORWF DECFSZ GOTO MOVLW BANKSEL MOVWF | PORTA,1 I2CCNT,1 LOOP3 B'00111111' TRISA TRISA | ; ; | Toggle ALARM RA7,6 Outputs, 5-0 Inputs (ALARM Hi Z to minimize pop) |
| BCF RETURN | STATUS,RP0 | ; | Select bank 0 |
| END |  |  |  |

; Creates "pip" PIP

LOOP3 LOOP4